The Presentation Manager Vector Fonts

by Charles Petzold

Let's begin with a question: What graphics programming

language stores fonts as a lines and curves (rather than

bitmaps), and thus allows fonts to be arbitrarily stretched,

rotated, outlined, filled with different patterns, or even

used as clipping areas?

One answer is obviously PostScript, Adobe Systems' page

composition language implemented on many high-end laser

printers (beginning with the Apple LaserWriter) and Allied

Corporation's Linotronic phototypesetters. Over the past few

years, PostScript has become the language of choice for

computer manipulation of fonts and text.

But an equally valid answer is GPI -- the Graphics

Programming Interface component of the OS/2 Presentation

Manager. As we'll see, GPI has facilities to do virtually

everything with fonts that you can do with PostScript.

The Trouble With Text

The display of text is always the most problematic part of a

graphics programming system. Unlike lines and polygons

(which are merely mathematical constructs), text is rooted

in a long tradition of aesthetic typography. In any computer

graphics system, the goal must always be to display text

that is as pleasing and as easy to read as a well-printed

book.

Yet, most computer output devices (such as video displays

and printers) are digital media. The subtly shaped and

rounded characters that comprise traditional fonts must be

broken down into discrete pixels for storage and then

reassembled on the output device. This often causes

distortions in the appearance of the text.

But one major advantage of using a computer for this job is

versatility:  We can use a wide variety of fonts in various

sizes and characteristics, and modify these fonts for

display.  The extent to which we can modify fonts is

dependent on the way in which the fonts are stored in

memory.

Images and Vectors

A font is generally stored in computer (or printer) memory

in one of two very different ways:

First, a font can be stored as an "image" or "bitmap." Each

character of the font is simply a rectanglar array of bits.

The 0 bits generally correspond to the background around the

character and the 1 bits correspond to the character itself.

Secondly, a font can be stored in a "vector" or "outline"

format. Each character is defined as a series of lines and

curves that enclose areas. The character is displayed by

drawing the outline on the output device and filling the

enclosed areas.

Image and vector fonts have distinct advantages and

disadvantages. Image font are always created for specific

font sizes and specific device resolutions. The size of a

particular image font cannot easily be changed. (For

example, enlarging an image font by doubling the rows and

columns of pixels often emphasizes the jaggedness of the

characters.) Also, image fonts cannot be rotated except

possibly by 90 degree increments.

Vector fonts are much more malleable. Because they are

defined as a series of lines and curves, vector fonts can be

stretched or compressed to any size and can be rotated to

any angle. Vector fonts are not tied to a particular output

device resolution.

In general, however, image fonts are more legible that

vector fonts. Various techniques are used to design image

fonts so they fool the eye into thinking the characters are

smoother than they actually are. Vector fonts --

particularly when displayed on low resolution devices and

scaled to small font sizes -- can be adjusted only by

mathematical algorithms, which currently are less capable

than human font designers. Another advantage of image fonts

is performance:  Vector fonts usually require much more

processing time to draw each character.

Most conventional laser printers store fonts as images,

either within the printer or in font cartridges. The printer

is restricted to specific font sizes and the characters

cannot be arbitrarily rotated. Much more versatile are the

fonts stored in PostScript-based printers. These fonts are

stored as vectors. PostScript fonts can be stretched or

compressed to any size, they can be arbitrarily rotated,

filled with various patterns, and used for clipping.

The GPI Fonts

The Graphics Programming Interface (GPI) of the OS/2

Presentation Manager can, of course, take advantage of fonts

that are stored in and supported by output devices such as

laser printers.

But GPI also includes its own support of both image and

vector fonts. The image fonts are expected, of course,

because they are particularly suited for low resolution

video displays and dot matrix printers. Image fonts are an

important part of most graphics programming systems (such as

Microsoft Windows GDI).

The addition of vector fonts in GPI is a real treat. GPI can

use these vector fonts with any output device. Thus, various

font techniques that previously have been restricted to

PostScript printers are now possible with other laser

printers and even the video display.

This article shows you how to work with vector fonts in GPI

and demonstrates many PostScript-like techniques. Quite

simply, the font support in GPI is nearly as strong as that

in PostScript. However, GPI has a serious deficiency that

I'll discuss at the end of this article.

OS/2 1.1 is shipped with three resource-only dyanamic link

libraries with a .FON extension. These are font files, and

the contents are shown in Figure 1.

In addition, the video display device driver (DISPLAY.DLL)

and printer device drivers may also contain fonts designed

specifically for the device. For example, the default

"System Proportional" font is stored in DISPLAY.DLL.

Important: If you want to use any of the fonts in the .FON

files, you must install the fonts from the Presentation

Manager Control Panel program. It is only necessary to

install one font from each of the three files, and you only

need do this once.

Each font has a face name. This is shown in quotation marks

in Figure 1.  Each of the image fonts is available in

several point sizes and for several output devices: the CGA,

EGA, VGA (and 8514/A), and IBM Proprinter. GPI can syntesize

variations of these fonts, such as italic or boldfaced

versions.

Vector fonts, however, need not be designed for a particular

output device and point size because they can be arbitrary

scaled. You'll note that italic and boldface versions of the

vector fonts are also included.

The vector fonts in GPI are equivalent to the Courier,

Helvetica, and Times fonts included in most PostScript

printers. However, because Helvetica and Times are

registered trademarks, these names are not used to identify

the vector fonts in GPI.

A little exploration of the font files will reveal that

vector fonts are encoded as a series of GPI drawing orders.

When drawing text with these fonts, GPI translates the

drawing orders into GPI functions, usually GpiPolyLine to

draw straight lines and GpiPolyFilletSharp to draw curves.

The VECTFONT Program

The VECTFONT program demonstrates the use GPI vector fonts.

For purposes of clarity, I've divided the program into

several modules.

The VECTFONT "Display" menu lists 16 options. The first

option (to display nothing) is the default. The other 15

options correspond to routines in the VF01.C through VF15.C

files (described below).  The VF00.C file contains some

"helper" functions used by the routines in VF01.C through

VF15.C.

VECTFONT creates a micro presentation space during the

WM_CREATE message using page units of PU_TWIPS. ("Twips" is

fabricated word standing for "20th of a point." A printer's

point size is 1/72nd inch, so page units correspond to

1/1440 inch.) VECTFONT then modifies the page viewport

rectangle so that a page unit corresponds to 1 point, which

(as you may know) is the default coordinate system in

PostScript.

Although VECTFONT displays output to the screen, vector

fonts are obviously more suited for laser printers. As you

will see, the appearance of the fonts -- even at 24 point

sizes -- is not nearly as good as the image fonts.

You'll also notice that several of the demonstration

routines in VECTFONT requires a few seconds to run. For

anything other than a demonstration program, you'd probably

want to use a second thread of execution so as to not hold

up message processing.

Selecting an Outline Font

To use an outline font in a Presentation Manager program,

you must first create a "logical font" and then select the

font into your presentation space.

The GpiCreateLogFont function creates a logical font and

associates the font with a local ID (a number between 1L and

254L that you select). This function requires a pointer to a

structure of type FATTRS ("font attributes") that specifies

the attributes of the font you want.

To create a vector font, most of the fields in this

structure can be set to zero. The most important fields are

szFacename (which is set to the face name of the font) and

fsFontUse, which is set to the constant identifiers

FATTR_FONTUSE_OUTLINE and FATTR_FONTUSE_TRANSFORMABLE,

combined with the C bitwise OR operator.

You may prefer using the CreateVectorFont function in VF00.C

for creating a vector font. The VF00.C file contains several

other helpful functions that are used by the demonstration

routines in VF01.C through VF15.C.

The CreateVectorFont function requires only the presentation

space handle, the local ID, and the face name:

CreateVectorFont(hps, lcid,

szFacename);

After you create a logical font (using either

GpiCreateLogFont or CreateVectorFont), you can select the

font into the presentation space:

GpiSetCharSet(hps, lcid);

The lcid parameter is the local ID for the font. After the

font is selected in the presentation space, you can alter

the attributes of the font with various functions described

below, obtain information about the font by calling

GpiQueryFontMetrics, GpiQueryWidthTable, and

GpiQueryTextBox, and you can use the font for text output

with one of the text functions such as GpiCharStringAt.

When you no longer need the font, you select the default

font back into the presentation space:

GpiSetCharSet(

hps,

LCID_DEFAULT);

and then delete the local ID associated with the font:

GpiDeleteSetId(hps, lcid);

In VECTFONT, I always use the identifier LCID_MYFONT for the

local ID. This is defined in VECTFONT.H as 1L.

Scaling to a Point Size

When you call GpiSetCharSet to select a vector font into the

prsentation space, the initial width and height of the font

are based on the GPI "character box," which defines a

character width and height in page units. The default

character box is based on the size of the default system

font. You can change the character box size by calling

GpiSetCharBox.

Very important: To get a correctly proportioned vector font,

it is necessary to change the character box size. Do not use

the default Generally you set the height of the character

box to the desired height of the font. If you want the a

vector font to have a normal width, you set the width of the

character box equal to the height. For a skinnier font, set

the width of the character box less than the height; for a

fatter font, set the width greater than the height.

If you're working in page units of PU_PELS, you must also

adjust the character box dimensions to account for

differences in horizontal and vertical resolution of the

output device. For this reason, it's much easier to work

with vector fonts if you use one of the metric page units

(PU_LOENGLISH, PU_HIENGLISH, PU_LOMETRIC, PU_HIMETRIC, or

PU_TWIPS). With these page units, horizontal and vertical

page units are the same.

For example, suppose you're using page units of PU_TWIPS.

This means that one page unit is equal to 1/20 of a point or

1/1440 inch.  After selecting a vector font into the

presentation space, you want to scale the font to 24 points.

You first define a structure of type SIZEF:

SIZEF sizfx;

The two fields of this structure are named cx and cy and are

interpreted as 32-bit FIXED numbers, that is, the high 16-

bits are interpreted as an integer, and the low 16-bits are

interpreted as a fraction.

If you want to scale the vector font to a 24 point height,

you can use the MAKEFIXED macro to set to the fields of the

structure like this:

sizfx.cx = MAKEFIXED(

24 * 20, 0);

sizfx.cy = MAKEFIXED(

24 * 20, 0);

Multiplying by 20 is necessary to convert the point size you

want to twips. Then call GpiSetCharBox:

GpiSetCharBox(hps, &sizfx);

After setting the character box, any character or text

dimensions you obtain from GpiQueryFontMetrics,

GpiQueryTextBox, and GpiQueryWidthTable will reflect the new

font size.

The ScaleVectorFont routine in VF00.C can help in scaling a

vector font to a desired point size. The function will work

with any page units, even PU_PELS. The second and third

parameters to ScaleVectorFont specify a point size in units

of 0.1 points. (For example, use 240 for a 24-point size.)

If you want a normally proportioned vector font, set the

third parameter to ScaleVectorFont equal to the second

parameter.

The VF01.C file shows how the functions discussed so far can

be used to display all the available vector fonts in 24

point sizes. You can run the function in VF01.C by selecting

the "24 Point Fonts" option from the VECTFONT "Display"

menu.

Arbitrary Stretching

Besides scaling the vector font to a specific point size,

you can also scale vector fonts to fit within an arbitrary

rectangle. For example, you may want to scale a short text

string to fill the client window.

The function shown in VF02.C shows how this is done. You can

run this function by selecting "Stretched Font" from

VECTFONT's menu. The function in VF02.C displays the word

"Hello!" in the "Tms Rmn Italic" font stretched to the size

of the client window.

The ScaleFontToBox function in VF00.C helps out with this

job. This function first calls GpiQueryTextBox to obtain the

coordinates of a parallelogram that encompasses the text

string. The character box is then scaled based on the size

of this text box and the rectangle to which the font must be

stretched. The QueryStartPointInTextBox function in VF00.C

determines the starting point of the text string (that is,

the point at the baseline of the left side of the first

character) within this rectangle. This point is used in the

GpiCharStringAt function to display the text.

Mirror Images

Besides scaling the font to a particular size, the character

box can also flip the characters around the horizontal or

vertical axis.

If the character box height (the cy field of the SIZEF

structure) is negative, then the characters will be flipped

around the baseline and displayed upside down. If the

character box width (the cx field) is negative, the

individual characters are flipped around the vertical axis.

Moreover, GpiCharStringAt will draw a character string from

right to left. It's as if the whole character string is

flipped around the vertical line at the left side of the

first character.

The function in VF03.C displays the same string four times,

using all possible combinations of negative and positive

character box dimensions. You can run this function by

selecting "Mirrored Font" from the VECTFONT menu.

Vector Fonts and Transformations

Unlike image fonts, vector fonts can be scaled to any size

and sheared and rotated to any angle. The matrix transforms

described in my article "OS/2 Graphics Programming

Interface: An Introduction to Coordinate Spaces" (Microsoft

Systems Journal, Volume 3, Number 4) affect vector fonts in

the same way they affect the display of other graphics

primitives.

In addition, GPI also supports several functions

specifically for performing transforms on vector fonts.

We've already seen how the GpiSetCharBox function allows

font characters to be scaled. The GpiSetCharAngle rotates

the font characters and the GpiSetCharShear performs x-

shear.

Character Angle and Rotation

By default, the baseline of the vector font characters is

parallel to the x axis in world coordinates. You can change

this by calling GpiSetCharAngle. This rotates the vector

fonts characters.

The character angle is specified using a GRADIENTL

structure, which has two fields named x and y of type LONG.

Imagine a line connecting the point (0,0) to the point (x,y)

in world coordinates. The baseline of each character is

parallel to this line. The direction of the text is the same

as the direction from (0,0) to (x,y).

You can also think of this in trigonometric terms. The

baseline of the text is parallel to a line at angle a

measured counterclockwise from the x-axis, where:

a = arctan (y/x)

and y and x are the two fields of the GRADIENTL structure.

The absolute magnitudes of y and x are not important. What's

important are the relative magnitudes and signs. The signs

of x and y determine the direction of the text string as

indicated in the following table:

x    y    Direction

-    -    ---------

+    +    To upper right

-    +    To upper left

-    -    To lower left

+    -    To lower right

The function in VF04.C uses the GpiSetCharAngle function to

display eight text strings at 45 degree increments around

the center of the client window. Each string displays the

fields of the GRADIENTL structure that is used in the

GpiSetCharAngle call.

In this example, the text strings begin with a blank

character so as not to make a mess of overlapping characters

in the center of the client window. The character angle does

not affect the interpretation of the starting position of

the string specified in the GpiCharStringAt function. If you

move your head so that a particular string is seen as

running left to right, the starting position still refers to

the point at the baseline of the left side of the first

character.

You can make the characters in a text string follow a curved

path by individually calculating the starting position an

angle of each character and displaying the characters one at

a time. This is what is done in VF05.C to display "Hello,

world!" around the perimeter of a circle.

The text string is scaled based on the circumference of a

circle that is positioned in the center of the window and

has a diameter half the width or height (whichever is less)

of the window. The GpiQueryWidthTable function is used to

obtain the width of individual characters and then space

them around the circle.

Character Shear

It is easy to confuse the character angle and character

shear. Lets look at the difference. The character angle

refers to the orientation of the baseline. Text displayed

with various character angles is rotated but otherwise not

distorted in any way.

The character shear affects the appearance of the characters

themselves apart from any rotation. Character shear by

itself "bends" characters to the left or right, but the

bottom of each character remains parallel to the x-axis. You

can use character shear to create oblique (sometimes

mistakenly called italic) versions of a font.

To set character shear you call the GpiSetCharShear

function. This function requires a pointer to a structure of

type POINTL, which has two fields named x and y. Imagine a

line drawn from (0,0) to the point (x,y) in world

coordinates. The left and right sides of each character are

parallel to this line.

The function shown in in VF06.C displays seven text strings

using different character shears.

You can run this function by selecting "Character Shear"

from the VECTFONT menu. Each string displays the x and y

values used in the POINTL structure to set the character

shear.

The character shear is governed by the relative magnitudes

and signs of the x and y fields of the POINTL structure. If

the signs of both fields are the same, the characters tilt

to the right; if different, the characters tilt to the left.

The character shear does not cause characters to be flipped

upside down. For example, character shear using the point

(100,100) has the same effect as (-100,-100).

The angle of the left and right sides of the characters from

the y axis is sometimes called the shear angle. In theory,

the shear angle can range to just above -180 degrees

(infinite left shear) to just under +180 degrees (infinite

right shear) and is equal to:

a = arctan (x/y)

where x and y are the two fields of the POINTL structure.

When you set a non-default character shear, the

GpiQueryTextBox function returns an array of points that

define a parallelogram rather than a rectangle. However, the

top and bottom sides of this text box are the same width as

for a non-sheared text string, and the distance between the

top and bottom sides also remains the same.

You can use character shear to draw an oblique shadow of a

text string. The function in VF07.C colors the background of

the client window blue, and displays the text string

"Shadow" twice.

The first call to GpiCharStringAt displays the shadow. This

is drawn in dark blue using a positive character shear. The

second call to GpiCharStringAt draws the characters upright

in red with a slightly smaller character box height. You can

run this function by selecting "Font with Shadow" from the

VECTFONT menu.

A Primer on Paths

To explore more capabilities of vector fonts, it's necessary

to become familiar with the GPI "path," which is similar to

a PostScript path. In GPI, you create a path by calling line

drawing functions between calls to the GpiBeginPath and

GpiEndPath functions:

GpiBeginPath(hps, idPath);

<... call line

drawing

functions ... >

GpiEndPath (hps) ;

This is called a "path bracket." In OS/2 1.1, idPath must be

set equal to 1L. The functions that are valid within a path

bracket are listed in the documentation of the Presentation

Manager functions.

The functions you call within the path bracket do not draw

anything. Instead, the lines that make up the path are

retained by the system. Often the lines you draw in a path

will enclose areas, but they don't have to.

After the GpiEndPath call, you can do one of three things

with the path you've created:

*    Call GpiStrokePath to draw the lines that comprise

the path. These lines are drawn using the

geometric line width, joins, and ends (discussed

shortly).

*    Call GpiFillPath to fill enclosed areas defined by

the path. Any open areas are automatically

closed. The area is filled with the current

pattern.

*    Call GpiSetClipPath to make the enclosed areas of

the path a clipping area. Any open areas are

automatically closed. Subsequent GPI calls will

only display output within the enclosed area

defined by the path.

Each of these three functions cause the path to be deleted.

Prior to calling any of these three functions you can call

GpiModifyPath, which I'll describe towards the end of this

article.

Normally, GpiCharStringAt and the other text output

functions are not valid in a path bracket. The exception is

when a vector font is selected in the presentation space.

When called from within a path bracket, GpiCharStringAt does

not draw the text string. Instead, the outlines of the

characters become part of the path.

This facility opens up a whole collection of PostScript-like

stylistic techniques that you can use with vector fonts.

Hollow Characters

Let's begin by calling GpiCharStringAt in a path bracket and

then use GpiStrokePath to draw the lines of the path.

GpiStrokePath has the following syntax:

GpiStrokePath(

hps, idPath, 0L);

In the initial version of the Presentation Manager, the last

parameter of the function must be set to 0L. When used to

stroke a path created by calling GpiCharStringAt, only the

outline is drawn and not the interiors. This creates hollow

characters.

The function in VF08.C uses this technique to display the

outline of the characters in the text string "Hollow." You

can run this function by selecting "Hollow Font" from the

VECTFONT menu.

You may want to display characters in one color with an

outline of another color. In this case, you must call

GpiCharStringAt twice, first to draw the interior in a

specific color, and secondly in a path bracket followed by a

GpiStrokeFont call to draw the outline.

The function in VF09.C does something like this to draw text

with a drop shadow.

The shadow is drawn first using a normal GpiCharStringAt

call. Another GpiCharStringAt functions draws the text again

in the current window background color at a 1/6 inch offset

to the first string. This is surrounded by an outline

created by a third GpiCharStringAt call in a path bracket

followed by GpiStrokeFont.

Quite similar to this is the creation of characters that

look like solid blocks, as shown in the function in VF10.C.

Eighteen character strings are drawn at 1 point offsets

using CLR_DARKGREEN. This is capped by the character string

drawn again in CLR_GREEN and the border in CLR_DARKGREEN.

Geometrically Thick Lines

At first, it may seem as if there is no difference between

drawing a line normally, like this:

GpiMove(hps, &ptlBeg);

GpiLine(hps, &ptlEnd);

and calling these same two functions within a path bracket

and then stroking the path, like this:

GpiBeginPath(hps, idPath);

GpiMove(hps, &ptlBeg);

GpiLine(hps, &ptlEnd);

GpiEndPath(hps)

GpiStrokePath(hps, idPath, 0);

There are, in fact, some very significant differences.

First, the line drawn with the normal GpiLine function has

what is called a "cosmetic" line width. The default width of

the line is based on the resolution of the output device. It

is a device dependent width for a "normal" line. The width

of the line does not change when you use matrix transforms

to set different scaling factors in the coordinate space.

Although GPI provides a function called GpiSetLineWidth to

change the cosmetic line width, this function is not

implemented in OS/2 1.1.

But a line drawn by stroking a path has a "geometric" line

width. This is a line width (in world coordinates) that you

set with the GpiSetLineWidthGeom function. Because this line

width is specified in world coordinates, it is affected by

any scaling that you set using matrix transforms.

Secondly, a line draw with GpiLine can have different line

types that you set with the GpiSetLineType function. These

line types are various combinations of dots and dashes. The

line is drawn with the current line color and the current

line mix.

But a line drawn by stroking a path does not use the line

type. The line is instead treated as an area that follows

the path of the line but which has a geometric width. This

area is filled with the pattern that you set with

GpiSetPattern, colored with the current pattern foreground

and background color, and the current pattern foreground and

background mix.

Third, a line drawn by stroking the path can have various

types of line "joins" and "ends." By calling GpiSetLineJoin

you can specify that lines meet with a rounded, square, or

miter join. GpiSetLineEnd lets you specify rounded, square,

or flat ends to the lines.

The function in the VF11.C file demonstrates the use of

geometrically thick lines filled with patterns to give the

letters a kind of "neon" look.

You can run this function by selecting "Neon Effect" from

the VECTFONT menu. The function strokes the path using

various geometric line widths filled with a PATSYM_HALFTONE

pattern and several colors. The outline of the font is

white, but surrounded with a halo of red.

A better effect could be achieved on devices capable of more

than 16 colors. In this case, you can use a solid pattern

but color each stroke with a different shade of red.

Filling the Path

When introducing paths earlier in this article, I mentioned

that you can do one of three things with a path: stroke it,

fill it, or use it for clipping. Let's move on to the

second: filling the path.

To fill a path, you call:

GpiFillPath(hps,

idPath, lOption);

This fills the path with the current pattern. Any open areas

of the path are automatically closed before being filled.

The lOption parameter can be either FPATH_ALTERNATE or

FPATH_WINDING to fill the path using alternate or winding

modes. For vector fonts, FPATH_WINDING causes the interiors

of some letters (such as "O") to be filled. You'll probably

want to use FPATH_ALTERNATE instead.

If the current area filling pattern is PATSYM_SOLID, then

the code:

GpiBeginPath(hps, idPath);

GpiCharStringAt(hps, &ptl,

cch, szText);

GpiEndPath(hps);

GpiFillPath(hps,

idPath,

FPATH_ALTERNATE);

does roughly the same thing with a vector font as a

GpiCharStringAt by itself. When using GpiFillPath you'll

want to set a pattern other than PATSYM_SOLID. (A bug in

OS/2 1.1 causes the current pattern to be reset to

PATSYM_SOLID during a path bracket in which GpiCharStringAt

is called. You can get around this bug by calling

GpiSetPattern after you end the path.)

The function in VF12.C uses GpiFillPath to display the text

string "Fade" eight times filled with the eight GPI shading

patterns (PATSYM_DENSE1 through PATSYM_DENSE8) and then

finishes by calling GpiCharStringAt outside of a path

bracket. You can run this function by selecting "Fading

Font" from the VECTFONT menu.

Clipping to the Text Characters

The third option after creating a path is to call

GpiSetClipPath:

GpiSetClipPath(hps, idPath,

lOption);

The lOption parameter can be either SCP_RESET (which equals

0L, so it's the default) or SCP_AND. The SCP_RESET option

causes the clipping path to be reset so that no clipping

occurs. The SCP_AND option sets the new clipping path to the

intersection of the old clipping path and the path you've

just defined in a path bracket. Any open areas in the path

are automatically closed.

You can combine the SCP_AND option with either SCP_ALTERNATE

(the default) or SCP_WINDING. As with GpiFillPath, you'll

probably want to use alternate mode when working with paths

created from vector fonts

The function in VF13.C calls GpiCharStringAt with the text

string "WOW" within a path bracket. This is followed by a

call to GpiSetClipPath. The clipping path is now the

interior of the letters. The function draws a series of

colored lines emanating from the center of the client

window.

The function in VF14.C uses a similar technique but draws a

series of areas defined by splines.

Modifying the Path

Between the call to GpiEndPath to end the path and the call

to GpiStrokePath, GpiFillPath, or GpiSetClipPath, you can

call GpiModifyPath. This function uses the current geometric

line width, line join, and line end to convert every line in

the path to a new line that encloses an area around the old

line.

For example, suppose that the path contained a single

straight line. After GpiModifyPath the path would contain a

closed line in the shape of a hotdog. The width of this

hotdog is the geometric line width. The ends of the hotdog

could be round, square, or flat, depending on the current

line end attribute.

Following the creation of a path, these two functions in

succession:

GpiModifyPath(hps, ID_PATH,

MPATH_STROKE);

GpiFillPath(hps, ID_PATH,

FPATH_WINDING) ;

is usually equivalent to:

GpiStrokePath(hps, ID_PATH,

0L);

GpiModifyPath and GpiStrokePath are the only two functions

that use the geometric line width, joins, and ends.

In theory, you can call GpiStrokePath after GpiModifyPath,

like this:

GpiModifyPath(hps, ID_PATH,

MPATH_STROKE);

GpiStrokePath(hps, ID_PATH,

0L);

This should do something, and it should be rather

interesting, but GPI usually reports that it can't create

the path because it's too complex.

Instead, let's look at GpiModifyPath followed by

GpiSetClipPath. The function in VF15.C is almost the same as

the one in VF13.C except that it sets the geometric line

width to 6 (1/12 inch) and calls GpiModifyPath before

calling GpiSetClipPath.

Note that the colored lines are clipped not to the interior

of the characters, but to their original outlines. By the

use of GpiModifyPath, the outlines of the characters have

themselves been made into a path that is 1/12 inch wide.

This is the path that is used for clipping.

Is It Enough?

I think it's clear that the facilities provided by GPI for

working with vector fonts equal -- and sometimes exceed --

those in PostScript. The GPI interface is very powerful and

very versatile.

Is that enough?

No, it's not. The implementation of vector fonts in GPI has

a structural flaw that still leaves PostScript the king.

Take a close look at the display of the "Helv" font. You'll

notice that the two legs of the "H" are different in width

by one pixel when they should be the same width. This is

undoubtedly due to a rounding error. It's obviously more

noticeable on a low-resolution video display than it would

be on a 300 dpi laser printer, but even on a laser printer

such errors will affect the legibility of the text.

Errors such as this do not occur with PostScript fonts.

PostScript fonts are true algorithms that are able to

recognize and correct any anomalies in the rendition of the

characters. In contrast, GPI fonts (which are encoded as a

simple series of polylines and polyfillets) are drawn

blindly without any feedback or correction.

So, while we can rejoice in what we have in GPI, there is

still the need to hope for improvement.

Figure 1

Dynamic Link

Library File   Image Fonts             Vector Fonts

------------   -----------             ------------

COURIER.FON    "Courier" (8, 10, and   "Courier"

12 points for CGA,     "Courier Bold"

EGA, VGA, and          "Courier Italic"

IBM Proprinter)        "Courier Bold Italic"

HELV.FON       "Helv" (8, 10, 12, 14,  "Helv"

18, and 24 points      "Helv Bold"

for CGA, EGA, VGA,     "Helv Italic"

and IBM Proprinter)    "Helv Bold Italic"

TIMES.FON      "Tms Rmn" (8, 10, 12,   "Tms Rmn"

14, 18, and 24 points  "Tms Rmn Bold"

for CGA, EGA, VGA,     "Tms Rmn Italic"

and IBM Proprinter)    "Tms Rmn Bold Italic"

Caption:  The OS/2 1.1 dynamic link library files that            Caption:

contain fonts. The font face names are shown in quotation

marks.