[Beta Release III][1-Feb-97]

-----------------------

AWACS 3.56 UPDATE NOTES

See the README file for new option switches and enhancements, including

leaving DTR as-is and more second-instance control switches.

-----------------------

AWACS 3.51 UPDATE NOTES

(see readme for more)

New and changed things:

- MONYiii.SYS is now required since all com info (caller ID and stats)

is accessed through it.  Either an MDA or a PM window, or both, can

still be used for output.

- The caller ID monitor (CID) now runs at time-critical priority, and it

may be switch to either OPEN or CLSD.  Once initialized, the caller ID

display is available no matter which application own the com port.  For

example, I run a PCBoard2 BBS and have it use  AT#CID=2  as part of the

modem init string.  On boot up, I run AWACS to init the modem/caller ID,

then close AWACS CID (awacs -close, or from the PM control box). Thereafter,

PCBoard owns the com port (as could BGFAX2, for example).  This differs

from previous versions where AWACS 3.51 now shows caller ID in real-time,

and not just when AWACS owns the com device/modem.

- New -baud:n option switch.  Default is 19200 and is fine.  Valid values

are 9600, 19200, 38400, 57600, and 115200.  This should be left at the

default unless there is a known need to change it.  Once closed, the

port may be reopened by another application, such as a BBS, at any baud,

though 115200 may be unecessary for most use.

- The -sneak option of 3.47 has been removed.  Better things to come.

SUPPORT:

- If you have any installation problems, or just don't know what you need

to install or how, write to info@40th.com (or support@40th.com if have

registered) and I'll set you up.

----------

AWACS 3.47

Copyright (C)1997 Cornel Huth

Part 1.  Overview.

Part 2.  Files included.

Part 3.  Installation.

Part 4.  Command switches.

Part 5.  Tips and traps.

Part 6.  Registering.

----------------

Part 1. Overview

----------------

AWACS is a basic system state monitor for display to either a MDA (mono,

digital) screen or to a PM window on the desktop (or both at the same

time).  Figure 1 shows a typical screen shot from the author's system.

The monitor stations are:

1.  Swap file size, and amount of swap used.

2.  RAM installed and the amount free and resident.

3.  Working set desired and minimum memory needed to run that instant.

The Swap used % is just that, the amount of used up swap file space.  For

RAM and working set, the % figure is relative to installed RAM.  Requires

MONYiii.SYS.  If an ISA-busmaster DASD controller is used (Adaptec 1542,

for example) a message pops up indicating that only the first 16MB of

memory will be used for user programs (unless the -noAbout is used).

The memory above 16MB is then added to the Swap file size.  This

is the way OS/2 works, and it's determined by your .ADD driver.

4.  AWACS control state:

a.  System status priority (LOW, MID, HIGH, HOLD) and check interval.

b.  Working set status priority (same as Sys) and check interval.

c.  Caller ID check priority (LOW, MID, HIGH, CLOSED) and check interval.

The check interval is in seconds.  The swap file is checked at an interval

synchronized to the System interval (see -swapX: AWACS switch).

The status priorities are thread priorities:

Set    CLASS   LEVEL

----  -------  -----

LOW   idle      31

MID   normal     0

HIGH  critical   0

HOLD  idle       1  (CID is "CLSD", since the com port is closed)

5.  CPU idle reserves.

The more dots the less idle time is available.  When pegged at 100%, this

means that the CPU is busy all the time.  This does not mean, necessarily

that the computer will respond slowly.  It only means that there's no idle

time.  For an accurate reading, no other pulse-like program should run at

the same time as AWACS.  If you were, for example, to run Warp's Pulse, the

AWACS CPU idle reserve status would indicate 50% or so, as it should since

there are now two idle+0 processes, each getting half the CPU (if nothing

else significant is running).  Warp's Pulse, however, is not so accurate.

This pulse is in 1-second intervals and is on the same priority as "Sys".

6.  A 24-hour clock, in large numerals.

7.  COM port status line:

This show the async device's state, in real-time.  This requires COM40.SYS.

The status line is available at any time, no matter which program has the

com port open if an MDA (mono) screen is used; the status is updated

directly by the async driver as it happens.

If an MDA (mono) screen is not being used, the PM window can be still be

updated every second if MONYiii.SYS has been installed with the -c

MONY switch.  If the mony driver is not installed, then the status line

can only be monitored when AWACS has the port open, which limits the

usefulness.

Therefore, if COM40.SYS is installed, and:

An MDA (mono) screen is used with the MONYiii.SYS driver, the COM status

line is updated instantly no matter who owns the COM port begin monitored.

If a PM window is used with the MONYiii.SYS driver installed using

the -c MONY switch, the COM status line is updated within a second, no

matter who owns the COM port begin monitored.

If a PM window is used -without- the MONYiii.SYS driver installed,

the COM status line is updated within a second, but only when AWACS owns

the COM port.

8.  Caller-ID:

If your modem has a caller-ID feature, you can use AWACS to see who's calling.

The top line is the most recent caller.  Up to nine calls can be logged if

the working set graphics is turned off (see -gc:0), or up to two calls if the

graph is displayed.  Not all calling areas provide caller-ID info (inbound

or outbound).  Contact your telco for details.  When an incoming call is from

an area that does not provide caller-ID, a "NOT AVAILABLE" is shown.  If a

caller blocks his caller-ID info, "NOT AVAIL/SNEAK" is shown.

Caller ID entries are buffered in a 16-item circular quueu, even if another

program has the com port (so long as the modem is inited with AT#CID=2).  The

buffered entries are displayed when AWACS is started with CID monitoring

enabled.

9.  Working set graphic:

The last minute or so of the working set usage is logged.  Use the included

allocmem.exe program to watch how it works.  The numbers along the right are

the memory level, in MB.  Amounts over 96MB are limited to the first 96MB.

Figure 1: Typical Screen Shot on either MDA or PM Window.

������������������������������������������������������������������������������Ŀ

�Swap: 32768      �Sys� LOW  3.0� COM2:02F8 115200 8N1 DCD  �  DSR CTS RTS DTR �

�used:     0 �  0%�Wks  LOW  2.0� FOYT A J        (713)555-1212 01/01/97 07:35 �

�����������������ĴCID CLSD  1.0�                                              �

� RAM: 48768      �������������Ĵ                                            48�

�free: 26052 � 53%�10   ::      �                                            42�

�rsdt:  5480 � 11 �     :: .    �                                            36�

�����������������Ĵ. : ::: :. . �                                      :::   30�

�wSet:           %�������������Ĵ                                      :::   24�

�wMin:            ��Ŀ�Ŀ �  �Ŀ�                                      :::   18�

�accs:  6412 �    �� �  �:��ų ��        ::                          ..:::   12�

�peak:  6900 � 14 ����  �   ���ٳ........::..........................:::::...06�

��������������������������������������������������������������������������������

AWACS 3.56                     1 day, 08:56                    Sat  1 Feb 1997

The last line shows the version, OS running time (to 999 days) and the date.

-----------------------

Part 2.  Files included

-----------------------

The following files are included in this AWACS package.  Specific installation

instructions follow in the next part.  This file you are reading is awacs.doc.

Not all files need to be installed.

1.  awacs.exe  (required)

AWACS.EXE is the Warp 3 or later, PM executable.  It provides a simple

interface to set the priorities and interval times.  All other settings are

made on the command-line, or from a program object's Settings' entry for

Parameters.

2.  monyiii.sys (required as of 3.51)

MONYiii.SYS serves multiple functions:  1) it's an MDA (mono) screen driver,

2) it provides an interface to the COM port status when using the PM window,

3) it provides the swap file stats, and as of 3.51, 4) it provides the

communication link between AWACS and the COM40.SYS driver.

See "Part 1.  Overview", "7.  COM port status line" for more.  It is

recommended that this be installed whether you have an MDA or not.  If MDA

(mono) hardware is not available, use the -c MONY switch.

3a.  theseus2.sys (optional)

3b.  awacst2.dll  (   "    )

THESUES2.SYS and AWACST2.DLL handle the monitoring of the working set state.

If used, both files must be used together; rename AWACST2.DLL to awacst2.dxx

if you do not intend to use them, or use the -noWks AWACS switches.

4.  awacst32.dll  (required only if no MDA hardware)

AWACST32.DLL handles the PM window updating.  If you intend to only use the

MDA (mono) display, this is not needed.  Either rename it or use the -noVio

AWACS switch.

5a.  com40.sys    (optional)

5b.  vcom40.sys   (   "    )

COM40.SYS handles the regular async duties (it is based on the Warp 4 COM.SYS

driver) and adds caller-ID and com port status monitoring.  VCOM40.SYS is a

standard replacement for VCOM.SYS and adds no features.  Baud to 115200 is

supported, and COM 1-4.  This can be used in Warp 3.0 or 4.0.

Even if you do not have a caller-ID modem, or caller-ID telco service, you can

still use COM40.SYS to monitor the com port (even without a modem attached).

See "Part 1.  Overview", "7. COM port status line" for more.

6.  monypage.exe   (optional)

Auxilliary program that is useful for programmers using a licensed version of

the Mony MDA driver.  It requires MONYiii.SYS.  It's purpose is to switch to

any of the up-to 8 mony video pages, or to disable the MDA (mono) screen.

7.  allocmem.exe   (optional)

Auxilliary program that allocates, commits, then touches memory pages.  Useful

for exercising the working set analysis features.

8.  zocevent.cm_   (optional)

A sample of using the -close and -open switches to control AWACS's com port

use.

---------------------

Part 3.  Installation

---------------------

What should you install?  You can install all the device drivers and

make use of all the DLLs, or you can use a subset:

o If you:

- Have MDA hardware and want to use it for the display, or

- Want to monitor swap file size and swap used, or

- Want to monitor the COM40.SYS port no matter who owns the port,

then install device=MONYiii.SYS

NOTE: As of 3.51, MONYiii.SYS must be installed.

o If you:

- Want to monitor a COM1-4 port, or

- Want to log caller ID name/number/time info,

then install device=COM40.SYS and device=VCOM40.SYS

o If you:

- Want to monitor system working set size,

then install device=THESEUS2.SYS and put AWACST2.DLL in LIBPATH=

o If you:

- Want to display the output to a PM window,

then put AWACST32.DLL in LIBPATH=

More specifically:

Copy AWACS.EXE to a directory in your PATH= statement.  Also copy ALLOCMEM.EXE

to the same directory (optional).  \OS2\APPS\ is a good location, if it's in

your PATH.

Optionally copy the AWACST2.DLL and AWACST32.DLL to a directory in your

LIBPATH= statement.  \OS2\APPS\DLL\ is a good location, if it's in LIBPATH.

Copy the four .SYS files to your boot drive's root directory, or those that

you wish to install.  If any of these files already exist, save your old ones

and use the ones provided in this package.

3.51  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE

|    As of version 3.51, AWACS requires the mony driver be installed.  It

|    is used as the pathway between AWACS and the COM40 driver.

If you want MDA (mono) screen support, or even if you don't, but you do want

real-time COM port status monitoring, add the line:

DEVICE=monyiii.sys  [-p#] [-q] [-c]

-p# is not required unless you plan on using AWACS on Mony page other than

the default (page 0).  This is not useful unless you have a Mony license

and are using page 0 already (e.g., -p8).

-q quiets the bootup message copyright.

-c should be used if you do NOT have MDA hardware, but you do want the

real-time COM status monitoring feature for your PM window.

A typical config.sys entry would be

DEVICE=G:\monyiii.sys -q     (if you have MDA hardware)

or, DEVICE=G:\monyiii.sys -q -c  (if you do NOT have MDA hardware)

If you want working set analysis, add this entry to config.sys:

DEVICE=G:\THESEUS2.SYS

If THESEUS2.SYS is not installed, working set analysis is not available.

Next, find your current async drivers.  They most likely will be named

COM.SYS and VCOM.SYS.  Comment their lines out and directly below each,

put the new ones in.  Be sure your MOUSE drivers come before.

DEVICE=G:\OS2\BOOT\MOUSE.SYS SERIAL=COM1

DEVICE=G:\OS2\MDOS\VMOUSE.SYS

...

rem rem DEVICE=G:\OS2\BOOT\COM.SYS

DEVICE=G:\COM40.SYS

...

rem rem DEVICE=G:\OS2\MDOS\VCOM.SYS

DEVICE=G:\VCOM40.SYS

In order for these new files to be loaded, you have to reboot.  Be sure to have

an OS/2 boot floppy nearby with an editor available in case you have to re-edit

you CONFIG.SYS file if, for whatever reason, your system no longer boots.

After the reboot, you are ready to start up AWACS.

AWACS' default state is for just about everything to be OFF.  You can control

everything from command-line parameters, covered in the next part.  The PM

control box lets you adjust priorities and intervals, and you can reset the

working-set and caller-ID sub-systems.

Note:  The PM window has a title bar and a minimize button, but these are

obscured.  To move the PM window, grab where the title bar would be and

drag the PM window about.  To minimize it, push where the minimize button

would be.  To restore the PM window, select the AWACS control box and click

anywhere on the control box (except the title bar line).

Part 4.  Command swiches

------------------------

It's recommended that you create a program object so that you can start up

AWACS with just the click-click of a mouse button.  To do this, open up the

TEMPLATES folder and drag a Program object onto the desktop.  The Settings

page pops up.  Fill out the path.  For the Parameter entries, consult the

following.  An example would be something like:

-com2 -bell -sys:3L -wks:2L -cid:1X -swapX:1 -cHeight:8

If you have MDA hardware, you probably want to add -noVio, and use -cls.

After a while you'll probably want to use -min and -noAbout.

-cHeight:n     Specifies the PM window character size.  For 1024x768

displays, the default is fine (8).  For 640x480, a

cell height of 14 is about right.  If you see "white"

space below the PM window's background, you need to

increase the cHeight value; if some lines are chopped

off, you need to decrease it.  See Figure 1. for how

the full display appears.  The range for n is 2 to 32.

For example: -cHeight:14

-com#          Specifies com port to use for both monitoring and CID.  Use

-com0 to disable monitor and any CID (the default is disabled).

For example, -com2  (no colon).

-istr:aaa      Specifies init string for caller ID, raw format.

The default is -istr:AT#CID=2

-bell          Sound bell on CID.

-noSys         Disable performance monitoring ("Sys" becomes ghosted).

-noWks         Disable working set analysis ("Wks" becomes ghosted).

Note:  If -noWks is specified. AWACST2.DLL is not loaded.

Swap file stats are also disabled if -noSys.

-noCID         Disables caller ID monitoring ("CID" becomes ghosted).

-noPulse       Disables pulse graphic.

-sys:TP        T=1 to 9 secs, P=h,l,m,x for hold, low, medium,

time-critical priority for System monitoring.  Finer

timing is available from the PM control box.  For

example, -sys:3L

-wks:TP        As -sys:TP

3.51  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE

|    As of version 3.51, AWACS runs the CID thread at time-critical

|    priority, and is either OPEN or CLSD (not LOW,MID,HIGH,...).

-cid:TP        As -sys:TP, except hold results in closed com port if hold

is true longer than 1.5 seconds

-gc:n          If n is 0, working set graphic is disabled and caller ID

logging extends to 10 entries (rather than 2).  If n is 1

(default), graphic uses . and : for markers.  If n is 2,

block/half-block are used.  For example, -gc:0  (also

results in caller ID to display the last 10 callers, not

just last two).

-rst:ddd       ddd is the working set calls to do before resetting.  The

default is 120.  No real need to change this, but the limit

is 250.

-page:n        The Mony (with MDA hardware) page to use for display.  n=0-7,

but must be valid mony page from DEVICE=monyiii.sys -pn.  This

page is 0-based, wherease DEVICE= line is 1-based. No effect

on PM window.  For example, -page:0 (the default).

-cls           Clears the MDA (mono) screen on startup.  No effect on PM

window.

-noAbout       Skips driver versions' box at startup; also skips ISA/16MB

busmaster driver limit notice (if applicable).

-min           Start the PM control box minimized.

-swapX:n       Rate at which the swap file status is check relative the "Sys"

rate.  The default is 2 (swap checked every other "Sys" check.

n=1 to 99, where n=1 causes a swap-check at every system check,

n=2 (default) every other system check.

-bg:n          PM window background color, n=0 to 15, default=0 (black).

-fg:n          PM window foreground color, n=0 to 15, default=6, yellow).

-hi:n          PM window intense color, n=0 to 15, default=14, bright yellow).

-noMony        Do not use the Mony MDA hardware.  Useful only if MDA is

actually installed.

-noVio         Do not use the PM window.  Also causes the AWACST32.DLL to

not be loaded.

Note:  Using both -noMony and -noVio results in no display.  Not very useful.

-minVio        Start PM window minimized.

Note:  The PM window has a title bar and a minimize button, but these are

obscured.  To move the PM window, grab where the title bar would be and

drag the PM window about.  To minimize it, push where the minimize button

would be.  To restore the PM window, select the AWACS control box and click

anywhere on the control box (except the title bar line).

3.51  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE

|

-sneak has been removed as of AWACS 3.51.  Later versions of AWACS

will offer even more useful 'tricks' than this.

-sneak:[nn[aaaa]]      If caller ID is available, anyone hiding his caller

ID info (i.e., "SNEAK/NOT AVAIL") will have the following

|            greeting:

|

|            o If -sneak: alone is used, your modem will attempt to

|              establish a connection with the caller (modem, or person!)

|              and if DCD is established within 24 seconds, the text BUSY

|              is sent to the calling mode and the connection closed.

|

|            o If -sneak:nn is used, the nn replaces the default 24 seconds

|              that AWACS allows for establishing a connection.  By setting

|              this to 01 (as in -sneak:01) you effectively pick up the

|              phone and then quickly disconnect (typically, no modem

|              squawk occurs).

|

|            o If -sneak:nnaaa is used, the nn (required) is as described

|              above, and the aaa replaces the default text message (BUSY)

|              with a message of your choice (60 bytes max).  For example,

|              "-sneak:20Who's there?".  Use " " around the argument if

|              spaces are used in the string.

|

|            For the best effectiveness, place the caller ID modem (machine

|            running AWACS) first in line on the phone line (i.e., nothing

|            between it and the wall outlet), and place any extensions,

|            answering machines, or other computer modems after it.  This

|            way, AWACS can control the modem line (disconnect it, etc.)

|            no matter if another device picks up.  This feature only works

|            for registered users, and in version 3.47 and later.  HIGH

|            priority at 1-second intervals for CID is recommended (e.g.,

|            -cid:1X).

|

NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE  NOTE

-baud:n        Initialize and use this as the port rate.  Valid values are

9600, 19200, 38400, 57600, and 115200.  The default is 19200

and should be left so unless there is a specific need.  Once

initialized, AWACS no longer needs the com port, though it

must be opened by somebody for caller ID to display (at any

baud rate).  The owner of the com device must send the

caller ID init str to the modem, too.

AWACS only runs one instance of itself.  Additional attempts just exit unless

one of the following, mutually-exclusive switches are used:

-close         Closes the open com port (see the included zocevent.cm_ file).

-open          Re-opens the previously-closed com port (see same file).

-exit          Exit AWACS.

5.  Tips and Traps

------------------

A)  If you are in other than 1024x768 the PM window may not appear to

fill the display properly.  Use -cHeight:14 for 640x480.  The range is

from 2 to 32.

B)  The working set analysis can use up lots of CPU time. or more specifically,

keep interrupts off for longer than you'd like.  If you have problems

downloading files (get CRC errors, for example), consider turning Wks to HOLD

while downloading.  The more main memory you have, the more work needs to be

done.  Also, the more active processes, the more work needed.  However, if

you have a very fast CPU, or not a lot of memory (32MB or less), you may not

notice.

C)  If you use LOW priority (as I almost always do), you may notice that when

the CPU is really busy, AWACS appears to stop.  It stops, indeed, because LOW

priority is the same as idle, and if there's no idle time, AWACS just doesn't

get any timeslices.  If this is a problem, for example, you need to monitor a

heavy CPU-use process, just bump up the priority to MID or HIGH.  Another

possibility is to set PRIORITY=ABSOLUTE in CONFIG.SYS (for this to make a

difference, you still need to use MID or HIGH).  You may also want to reduce

the interval:  1.0 seconds is the lowest.  I use Sys:3L, Wks:2L, CID:1X and

SwapX:1 most of the time.  (1X for CID if -sneak:).

D)  If the Pulse seems to no longer respond, except in drastic changes (0%

most of the time, but if you move a big window, then 30%, but just for a

second), then you just exit and restart AWACS.  I've seen it, though it

doesn't happen often (not more than once a month, and not in several months).

E)  As of AWACS 3.45, the swap monitor is much faster (-swapX:1 is fine).

6.  Registration

----------------

Why register?  Because if you don't, you can only use AWACS for 30 minutes at

a time, and only for the first 2 hours after booting.   If you want to

get rid of those time limits you need to register.  The cost is $17.50 plus

$2.50 shipping and handling.  Payment may be check, money-order, or cash.

Payment must be in US funds drawn on a US bank.  Send $20 to:

Cornel Huth

6402 Ingram Rd

San Antonio TX 78238

USA

Questions on registering?  Contact info@40th.com  or call the 40th Floor BBS

at 1(210)684-8065.   If you have registered AWACS 1.x already (before 1997)

contact me and I'll get you a free 3.0 registration.  For those that only

need an MDA (mono) version, AWACS 2.10 is still available at no cost (at most

OS/2 BBSes and internet sites).  Sorry, no credit cards (it's only $20! but

you get a (filled) disk, receipt, and support).  If you really can't do it

any other way, contact bmtmicro and I'm sure they can do something about your

credit card order.  Ask them about "AWACS by Cornel Huth" ($20-).

^Q