A Sample OS/2 IBM 16-bit Token-ring Driver Notes:

-------------------------------------------------

Furnished by DWB Associates, Inc. Copyright (c) 1993.

The IBM 16 Tokenring adapter is a standard ISA BUS 16 bit network card

compatible with Tokenring topology networks. The Driver is designed to

operate under Microsoft LANMAN 2.1 for OS2 1.3 operating system.  IBM Lan

IBM Lan Server running under OS2 2.0 are supported. These notes cover

certain details of operation for building the executable from sratch

as well as installing and using the driver software on the above

mentioned system.

About this Product:

-------------------

Three files exist in the \IBM16\PRODUCT directory that can be used to install

the Sample OS/2 IBM 16 bit Token-ring driver immediately without any further

development procedures.  The three files are the IBM16.OS2, IBM16.NIF, and

PROTOCOL.INI files.  The section titled "Installation Disk Notes:", which

follows the next section, will help you to get started on installing and

testing this sample package right away.

If you are interested in further development of the source files and using

the MAKEFILE associated with this package, the following section titled

"Source Build Notes:" should be of interest.  This section will help you

to develop your own driver for the IBM TR 16 card.  See also the README2.DOC

file found in the directory \IBM16 for more information about this and using

the MAKEFILE for custom development.

Source Build Notes:

-------------------

Source code for the driver is included for those developers interested in

modification for their own purposes or to see how a driver is structured

for the IBM 16 Tokenring architecture.  The source code is found in the

directory titled \IBM16\SOURCE.

The MAKEFILE, found in the \IBM16 directory, can be used to build the

driver.  You can run the MAKEFILE by typing NMAKE at the command line

in the same directory.  The standard Microsoft Toolkit files are assumed

for building the driver.  The following is a list of environment variables

used by MAKEFILE.

ENVIRONMENT VARIABLES:

SRCPATH = path to where the source code resides

OBJPATH = path to the object files

PRODPATH= path to the product files: ibm16.os2, ibm16.nif, and protocal.ini

INCPATH = path to include files

ERRPATH = path to the warnings/error files created during compilation

TYPE THE COMMAND:

NMAKE    - At the command line prompt.  This makes the OS2 version of

the Sample OS/2 IBM 16 bit Token-ring driver.

Installation Disk Notes:

------------------------

INSTALLING THE DRIVER:

There are four different changes neccesary to be made in order to install

the Sample OS/2 IBM 16 bit Token-ring driver.  Three of these involve putting

new files into place and the other one involves making changes to an existing

file.

The three files that need to be put into directories on the hard disk of

the machine you will be using are the IBM16.OS2 file (the driver), the

IBM16.NIF file (the network information file), and the PROTOCOL.INI file

(the protocol file).  All three of these files can be found with this sample

driver package in the \IBM16\PRODUCT directory on the diskette that has this

README.DOC file.  The existing file, which is found in the root directory on

the system you will be using to test, that needs to be updated is the

CONFIG.SYS file (the system configuration file).  Use the instructions that

follow to make these changes:

1.   The IBM16.OS2 driver should be placed in a subdirectory that you create

to the \LANMAN\DRIVERS\TOKENRNG directory on the system you want to use

to test the driver.  Whatever name you give to the subdirectory, be sure

to remember it.  You will need the path of the driver to list it as a

device in the CONFIG.SYS file.  The change to CONFIG.SYS is discussed

in step 4. below.

2.   The IBM16.NIF file is placed in the \LANMAN\DRIVERS\NIF directory.

3.   The PROTOCOL.INI file is placed in the \LANMAN directory.  You may want

to take care to save any existing PROTOCOL.INI file that may already

exist in the \LANMAN directory to use in another application.

4.   Find the CONFIG.SYS file in the root directory of the system you will

be doing your driver testing on.  This file needs to have a change made

to it in order to run the driver.  The following line needs to be added:

DEVICE=C:\LANMAN\DRIVERS\TOKENRNG\<your subdirectory>\IBM16.OS2

Take note of a couple of things.  First of all, if the drive your

machine normally boots up on is not the C: drive, put in the name of

the drive in place of the C: shown above.  Secondly, be sure to type

in the name of the subdirectory you created to place the driver in where

it says <your subdirectory>.  This line should be added towards the end

of the contents of the file CONFIG.SYS.

5.   Reboot the system with the IBM 16 Token-ring card installed.