Information file for IBM Auto LANStreamer PCI Adapter NDIS 2.0 MAC Device Driver

------------------------ Information About This Release ------------------------

IBMMPC.OS2 Version 4.57.00

This device driver supports the following IBM Streamer Family adapters:

- IBM Auto LANStreamer PCI Adapter

------------------------------ General Information -----------------------------

The message files for the IBM Streamer Family NDIS MAC Driver are LTC.MSG and

LTCH.MSG.  The messages and their associated help text refer in general to the

adapter name as IBM Streamer Family adapter.

The IBM Streamer Family NDIS MAC Driver supports Promiscuous Copy.

When configuring multiple IBM Streamer Family adapters using the system

Reference Diskette, you are no longer required to set all of these adapters

to the same System Interrupt Level in order to operate with this version of

the NDIS MAC Device Driver (although non-PCI adapters will continue to work

properly if set to the same level).

Support for OS/2 Computers with more than 16 Megabytes of Memory:

The IBM Streamer Family adapters use bus mastering to transfer data

between the computer and the adapter.  For 16-bit adapters, or any

adapter placed in a 16-bit system slot, only 16 megabytes (MB) of

system memory can be addressed.  For this reason, the IBMMPC.OS2

device driver will automatically configure and use local buffers

allocated below this 16 MB addressing limit.  Frame data located above

16 MB will be copied into local buffers for frame transmission so

that it can be accessed by the adapter.

************************** Changes to the Publication **************************

------------------- NDIS MAC Driver Parameters in PROTOCOL.INI -----------------

The Network Information File (NIF)  used with the IBM Streamer Family NDIS

MAC Driver is IBMMPC.NIF. It contains configuration and tuning parameters

that can be adjusted by the user.  These parameters are used in the

PROTOCOL.INI file. Look at the NIF file for Help text available on each

parameter.  Additional information on some parameters that is not included

in some editions of the publication is provided here.

LoopBack -- will default to NO for an adapter that is either a

16 bit card or in a 16 bit system bus slot, AND is used for

Ethernet or 16 Mbps Token Ring operation.  For all other cases,

with the exception of token ring full duplex, LoopBack defaults

to YES.  If the EnableTRFDX parameter (see below) is set to YES,

this LoopBack parameter must be set to match the loopback capability

of the attached token ring full duplex switch, and will default to

NO if not specified in this case.

Enet20UTP -- enables 20 Mbps Ethernet operation in an IBM Streamer

Family adapter that supports Ethernet.  The adapter must be

attached with a UTP cable to a switched Ethernet concentrator

that supports full duplex operation for this function to be

activated.  When this function is activated, the adapter loopback

mode will always be forced off, regardless of the setting of the

LoopBack parameter.  The Enet20UTP parameter is ignored for token

ring operation.

UseVDS --  allows the device driver to build the necessary data structures

for use with Virtual DMA Services (VDS).  Setting this parameter to

"YES" is a requirement when using the IBM Streamer Family NDIS MAC

driver with either Microsoft Windows(**) or Microsoft Windows For

Workgroups(**) in enhanced mode.  It may also need to be set to

"YES" when using some memory managers that support VDS.

LLCOnly -- indicates whether only LLC frames should be copied by the

adapter when it is running in promiscuous copy mode.  If this

parameter is set to "YES", then no MAC frames will be copied while

in promiscuous copy mode.

DataRate -- used for configuring the ring speed of the IBM Auto LANStreamer

PCI adapter.  It can be set to "AUTO", "16" or "4", with "AUTO" being

the default value if none is specified.  Note that this parameter is

only used by IBM Streamer Family PCI adapters.

EnableTRFDX -- configures the device driver to attempt to use the adapter

in token ring full duplex mode.  The adapter must have full-duplex-

capable microcode installed, and must be connected to a full duplex

switch before this mode of operation is attempted.  Also note that

for proper token ring full duplex operation, you must set the value

of the LoopBack parameter to match the frame loop back capabilities

of your full duplex switch.  The EnableTRFDX parameter is ignored for

Ethernet mode (see the Enet20UTP parameter description above).

Maximum Frame Size Supported:  the maximum frame size supported by the NDIS

MAC driver is determined by the SMALLEST of the following values:

- MaxTxFrameSize setting in PROTOCOL.INI

- (MinRcvBuffs x SizWorkBuf) PROTOCOL.INI parameters, less 4 bytes

- (10 x SizWorkBuf) PROTOCOL.INI parameter

- Largest frame permitted by the physical LAN medium (approximately 4500

bytes for 4 Mbps and 18000 bytes for 16 Mbps token rings, and 1514

bytes for Ethernet)

Microsoft Windows For Workgroups(**) Users:  when entering or changing the

value of the Network Adapter Address parameter for the IBM Streamer Family

adapter, you must enclose the address string in double quotes.

Example:   "xxxxxxxxxxxx"

---------------------- New/Changed Device Driver Messages ----------------------

Some device driver messages were added or changed after some editions of the

adapter publication were published.  The affected messages are shown below.

LTC0013 -- Slot nn:  Invalid IBM Streamer Family adapter configuration

detected;  interrupt 0 not allowed.

Explanation:  The device driver has detected that interrupt level 0 was

assigned by the system to the IBM Streamer Family adapter

located in Slot n.  This interrupt level is invalid, and was

probably assigned by the system automatically, especially for

PCI adapters.

User Action:  Reconfigure the system slot to use a valid interrupt

level for this adapter.

LTC0023 -- Slot nn:  The adapter is configured for full duplex, but could

only be opened in half duplex mode.

Explanation:  The EnableTRFDX keyword in PROTOCOL.INI was set to "YES",

indicating that full duplex token ring operation is desired, but

the adapter was not able to open in full duplex mode.  Instead,

it opened successfully in half duplex.

User Action:  This is an informational message only.  If full duplex

operation is desired, you should verify that the adapter is

properly connected to a token ring full duplex switch and that

the switch is operating in full duplex mode.  If you are not

interested in token ring full duplex mode, change the setting

of the EnableTRFDX keyword to "NO" in PROTOCOL.INI.

LTC0034 -- Slot n: Network failure or beaconing was encountered

while inserting into the network.

Explanation:  The token ring network is not functioning,

or is currently in a beaconing state.  Another possibility is

that the token ring cable is not connected to the adapter

card, or the adapter is configured for the wrong speed.

User Action:  Make sure the token ring cable is attached to

the appropriate connector on the adapter.  Verify that the

adapter is properly configured for the ring speed (data rate)

and media type.  Reboot your computer.  If this problem still

exists, contact your network administrator.

LTC0037 -- Slot n: Data rate of the Token Ring does not match the

data rate configured for the adapter.

Explanation:  The data rate of the network is different from

the data rate configured for the IBM Streamer Family adapter.

That difference was detected when the adapter attempted to

insert into the network.

User Action:  For MCA adapters, use the reference diskette

that came with your computer to ensure that the adapter is

configured at the same speed as the Token Ring, or is

configured for autosense mode. For ISA adapters, use the

appropriate configuration program. For PCI adapters, add or

modify the DataRate parameter in PROTOCOL.INI to the data

rate of the network to which the adapter is connected.

LTC0038 -- A new adapter cannot be opened in AutoSense mode if it

is the only station on the network.

Explanation:  While attempting to open a new adapter in Token

Ring mode using the AutoSense configuration option, it was

determined that the adapter was the only station on the ring.

The adapter will not be allowed to open in order to prevent

possible network disruptions.  (NOTE:  some adapters use the

term 'autospeed' instead of 'autosense' to mean the same thing).

User Action:  Reboot the computer at a later time, when at

least one other station is on the ring.  As an alternative,

reconfigure the adapter for manual mode, specifying the

correct speed of the Token Ring to which the adapter is

connected, then restart the computer.

LTC0040 -- Slot nn:  Using local transmit buffers to access data above

16 megabytes.

Explanation:  The device driver has detected more than 16 megabytes

of RAM in the computer, and the adapter is either a 16-bit

adapter or is located in a 16-bit system slot.  The device

driver will copy any frame data located above the 16 MB

address limit to local transmit buffers.

NOTE TO OS/2 2.0 USERS:  Early versions of OS/2 2.0 will

only indicate that at least 16 MB of RAM exist in the

computer.  If your computer has EXACTLY 16 MB of RAM,

you will see this message, but there will never be

any data above the 16 MB addressing limit to copy into

local buffers.

User Action:  This is an informational message only.  No action is

necessary.

LTC0041 -- Slot nn:  Overriding MAXTRANSMITS in PROTOCOL.INI with a

value of xx.

Explanation:  The amount of local memory needed to hold all possible

queued transmit frames, MAXTRANSMITS * MAXTXFRAMESIZE, is

greater than 128 kilobytes.  The device driver has reduced

the MAXTRANSMITS value to allow the maximum number of

transmit frames to fit within the default 128 kilobytes of

local buffer space.

User Action: Ensure that the values specified for MAXTRANSMITS and

MAXTXFRAMESIZE are accurately specified in PROTOCOL.INI.  If

not specified, they should both be added with appropriate

values.

The default values for these parameters in OS/2 are

MAXTRANSMITS = 31 and MAXTXFRAMESIZE = 18000, which exceeds

the 128 kilobytes allocated by the device driver for the

local transmit buffer space.  These local transmit buffers

are needed when more than 16 megabytes of system memory is

detected, and this IBM Streamer Family adapter is unable to

access above 16 megabytes.

A new PROTOCOL.INI parameter, LOCALBUFFSEGCNT, can be used

to change the amount of storage used for the local transmit

buffers.  It can be set from 1 to 16, and defaults to 2 if

not specified.  Note that 64 kilobytes of system memory are

used for each local buffer segment configured with this

parameter.

Example:   LOCALBUFFSEGCNT = 2

LTC0042 -- OEMHLP could not be accessed.

Explanation:  The IBM Streamer Family adapter device driver was

unable to determine the amount of system RAM in the computer.

User Action:  Contact your customer service center.

LTC0047 -- Could not determine the system I/O bus type

Explanation:  The IBM Streamer Family NDIS MAC Driver was

unable to determine the type of I/O bus designed into the

computer system.  It is looking for a Micro Channel, ISA,

EISA, or PCI bus.

User Action:  Make sure your system is based on a Micro

Channel, ISA, EISA, or PCI bus and contains an industry

standard BIOS. Contact your customer service center.

LTC0057 -- Slot nn:  IBM Streamer Family adapter opened for:  Token Ring,

xx Mbps Full Duplex.

Explanation:  Denotes that the adapter was opened in token ring full

duplex mode.

User Action:  This is an informational message only.  No action is

necessary.

LTC0058 -- Slot nn:  IBM Streamer Family adapter has entered promiscuous

copy mode.

LTC0059 -- Slot nn:  IBM Streamer Family adapter has exited promiscuous

copy mode.

LTC0060 -- Slot nn:  IBM Streamer Family adapter has received a remove LLC

frame.

Explanation:  These three messages provide information for an adapter

that enters and exits promiscuous copy mode.

User Action:  These are informational messages only.  No action is

necessary.

LTC0063 -- Slot nn:  A protocol error occurred while attempting to open

the adapter in full-duplex mode.

Explanation:  The adapter is configured for token ring full duplex

operation, but encountered a problem when attempting to connect

to the full duplex switch.

User Action:  Verify that your full duplex switch is functioning

properly and that your adapter is properly connected to it.

If this problem persists, contact your customer service center.

(**) Trademarks

---------------

IBM is a trademark of International Business Machines, Inc.

Streamer is a trademark of International Business Machines, Inc.

LANStreamer is a trademark of International Business Machines, Inc.

EtherStreamer is a trademark of International Business Machines, Inc.

Microsoft Windows is a trademark of Microsoft, Inc.

Microsoft Windows For Workgroups is a trademark of Microsoft, Inc.